Compiled by : Nauroze Hoath

Executive Summary

A prototype system that implements the microblogging service Twitter onto an Arduino microcontroller, thereby allowing long distance communication with the microcontroller. The project’s goal was to make use of this configuration and operate various appliances at home.

The system was implemented using an Arduino Uno R3 microcontroller, which was powered through a computer by the means of a serial connection.  The control hub of the Arduino Uno was the Atmel ATmega328p microcontroller. The ATmega328p chip consisted of a 32KB ISP flash memory and 2KB SRAM [1], this memory was realized to be inadequate in order to allow the full functionality of the project, hence the java based compiler Processing was also used to run this system. This gave flexibility in memory usage and also acting as an intermediate to connect the Arduino to the Web. The system also consists of a mechanical relay from PowerSwitch Tail that enables appliances that operate at 120v AC. Additionally, the project had an infrared sensor-emitter pair, a TMP37 temperature sensor, a 12V Dc powered PC fan and a red LED, demonstrating the distinctiveness of the project.

The project was split into 4 parts; two members worked on setting up Twitter and program it in accordance with the Arduino, one worked on designing a circuit that controls a DC fan through a 12V supply, another member had to set up the IR emitter/receiver pair circuit and the final group member had to set up the PowerSwitch Tail relay in addition to calibrating the temperature sensor.

The working of the system went as follows:

- A user would make a request to operate/check status of a particular appliance.

- This would be done through the Twitter app or webpage.

- The request is sent to the java compiler – Processing.

- The compiler recognizes it and sends a request to the Arduino to operate the particular appliance.

- Arduino turns on the particular appliance. (Unless the request is perform a status check, then it would read the status of the corresponding appliance.)

As this is a prototype, only one 120V AC appliance in the form of a Lamp is operated. Additional appliances can also be added, if more relay switches are plugged into the system. To exhibit the diversity of the system the rest of the appliances are modelled by smaller scale devices such as the fan and the sensors.

Introduction

Background:

Twitter is a social networking, micro-blogging and real-time communication service. It allows its users (and businesses) to exchange bit-size information with one another and the world. These bit-size messages are known as ‘tweets’ and they have a standardized character limit of 140. Ever since its debut in 2006, it has transformed into a large-scale open networking environment and ecosystem, where people, developers and businesses flourish together with their ideas and innovations. Since Twitter has become such an integral part of people’s lives, the group decided to construct a system where its users are allowed to integrate their homes into their everyday tweeting habits. This has created a revolutionary platform that brings about a unique and low-cost approach to home automation.

Benefits of Tweet-A-Home:

There are a rising number of home automation services, but they tend to come on a hefty price tag. An individual would be paying around $1000 to $3000 on wiring installations alone [2], and then comes the additional charges in the form programming labour and monthly subscriptions. The idea of saving on the bills by installing such a system becomes redundant when the pay-off period becomes lengthy. The Tweet-A-Home system is not only a smart solution, but it is an economic one too. The system takes advantage of Twitter’s free usage and pre-existing platform, thereby allowing developers to not worry about creating an interface. Additionally the developer doesn’t need to host and maintain servers, as this is already provided by Twitter. This lowers the production costs and thereby lowering purchasing cost by the user as well. As for installation costs, the system is designed in a ‘plug and play’ fashion, allowing the user to plug in devices as they like, giving them the ‘DIY’ experience.

System Overview:

The layout in Figure 1 presents the structure of the project.  The system will recognize various commands given by the user through a mobile device, in the form of tweets. In response it will operate different devices corresponding to the request sent. The following appliances and sensors will be modelled and demonstrated:

- A 120V AC Lamp

- A 12V DC powered fan – Representing any cooling system at home.

- An LED – Representing a heater

- Temperature Sensor

- An IR Emitter-Receiver pair. – Representing intrusion motion sensors.

Figure 1 - System Overview

Technical Details

The five technical design aspects of this project are discussed in this section and they are as followed:

- Twitter and Processing Set up

- Pairing Arduino with Processing

- Powering the DC Fan Circuit

- PowerSwitch Tail and Temperature Sensor Calibration

- Intrusion Alert System

Twitter and Processing Set up

Twitter has a dedicated webpage for developers and HTML programmers, called Twitter Developers. [3] This page allows users to create accounts that can access the ‘back door’ of Twitter which a regular account is incapable of doing. Twitter provides an API to these developers, allowing them to get Twitter feeds on their websites and also to send automatized habitual tweets. This idea then had to be implemented onto the Arduino micro-controller instead of a website. Hence to work with Twitter without the use of HTML, an intermediary library had to be used. The Library that was picked was the Twitter4J library [4], due to its ability to function in Java and communicate with Twitter’s API, as a result it simplifies the coding process.

This branch of the report walks through the set-up of the Twitter account, the implementation of the Twitter4j library onto the java based compiler Processing and finally building the code that allows Processing to communicate with the Arduino and the rest of the system. Note that all the codes in this section are done on Processing IDE. 

Twitter Account Registration:

The account was created on Twitter Developers page as discussed earlier. It was named ‘Arduino_3907’ and set up as a private account. This was done primarily to avoid other Twitter users to view information regarding the system, thereby preventing any leak of sensitive information.

Once the account was registered, the next step involved registering an ‘app’ for our account. The app was named ‘Arduino_3907’ just as the name of the Twitter account. The purpose of the app was that it integrates itself into the account, thereby allowing automated reading and posting of tweets. In essence, the app enabled a communication link between the Twitter account and Twitter’s API.

The app also required some initial configuration, the primary one was setting the Access level of the app. The choices given were between: Read only, Write only and Read & Write. Since the Tweet-A-Home system reads user sent commands and writes feedbacks, the evident choice was Read & Write.

When the app was set up, it generated a set of four authorization keys (Figure 2). The authorization keys are unique to every app made and it allows the developer have access control over their account through HTML or as it is in the case of this project, in Java. It essentially allows the app to make authorized calls to Twitter’s API and authenticates the account.

These keys are:

- Consumer Key

- Consumer Secret

- Access Token

- Access Token Secret

Figure 2 - Authorization Keys - For security reasons the keys are partially censored.

Processing Set Up:

The Twitter4j library was installed into Processing’s library folder; this allowed the IDE to easily import the library into the compiler. The purpose of the library is to allow Processing to gain access to the functions available on Twitter’s API. The library is imported by calling in the desired classes, as it is shown in Code 1. In addition to the classes imported for twitter, the java.util and processing.serial classes were called in as well. Java.util enables the general java functions and processing.serial enables the functions responsible for pairing Arduino IDE with Processing IDE.

Code 1 - Importing libraries

The first step in the coding process is to create a method/function that will allow the program to connect to Twitter. This is a standardized way of creating this connection; it involves three variables created through classes from the library and the authorization keys generated for the app (Figure 2). The name of the created function is connectTwitter() and is shown in code 2. 
(The AccessToken keys are hidden for security reasons in the code)

As it is with every program, there has to be a setup function. It is in this function a serial link between the Arduino and Processing is created, this is done by connecting Processing to the same port that the Arduino is connected on. ‘Serial.list’ is an arraylist of all the available ports on the machine, the port shown in the brackets in Code 3 is of the location ‘1’. This is because on that particular machine/computer the Arduino micro-controller was plugged into the port COM1, hence Processing had to be connected to COM1 as well. In addition to that connectTwitter() function was also used in here to establish a connection when the program is run.

With these two functions made, the connection to twitter was now complete; the next step was to get the recent tweets, which are posted as commands, on the Twitter profile.

A custom made function was made to get the tweets through the getUserTimeline() method which pre-existed in the library. This newly created function was named getTweets(), the reason it was created was because the getUserTimeline() method extracted all the tweets posted on the twitter page and returned it as an Array of tweets, but for the purpose of this project only the most recent tweet was needed. So this was done as demonstrated in the sketch in Code 4, where the method ‘statuses.get(0)’ extracts the most recently posted tweet.

Also seen on the sketch in Code 4 is that the commands (to turn on a particular device in this case) are also performed by this function. In essence, it extracts the tweet, compares it with the string that is required and once matched it sends a character to Arduino to turn on a device (see Figure 3). The character is 1 in this case, but it could be represented by any character.

Figure 3 - Processing to Arduino layout.

The next step in programming Processing was to set up a function for posting tweets. For the project it is required to post tweets automatically whenever a certain command has been met or if one of the sensors (temperature sensor, IR etc..) wants to post data on twitter. The name given to this function is postTweet() and is demonstrated in the sketch shown in Code 5.

Both Code 4 and Code 5 contains a catch function, this was added because it extracts error messages from Twitter’s API. So if the servers are down or have any connection issues, this function will display the corresponding error.

The final part of setting up Processing was constructing the function to get responses from Arduino. These responses came in the following form:

- Temperature readings when requested.

- Intrusion alert message when the IR is disturbed.

- A confirmation after an appliance/device is turned on/off.

- A feedback message after an issue to check the status of a appliance/device is made.

This was performed by creating a string called ArduinoMsg which checks for messages that Arduino posts on its serial monitor using the readStringUntil method. An additional variable was made called ArduinoTemp  , this variable temporarily stores the last received message in order to make a comparison with ArduinoMsg , this ensures the message isn’t displayed more than once.

The method was called responseCheck() , as shown in Code 6, and it is called when the program is executed.

All the different chunks of codes shown in this section were eventually merged together and the finalized program for Processing is shown in Appendix B.

Pairing Arduino with Processing

This section shows how the Arduino code was planned and set up. It addition it implements the codes for the Infrared pair and Temperature sensor, which are derived later in this report.

How It Works

The Arduino will serially receive instructions in form of single character parameters  from Processing. The Arduino will then compare these characters with a set of commands and perform the command that correspond with each character. These commands are what set the home appliances on or off. After each command is performed, the Arduino tweets back to the user that the command has been done.

The Code Developed

The code was divided into three main parts. The first part controlled the basic on and off functionality, the second part was used to operate the IR system and the third part was used to operate the temperature sensor.

At the very start of the code, all variables that were being used in different functions were initialized along with a set of strings that would be used when the commands are performed. The purpose of each of these strings was to be sent to Processing after each command was done. Once the string is sent, Processing would then proceed to tweet these strings back to the account to notify the user that a specific instruction was done. The strings used are shown in Code 7 below.

Code 7: Strings used for tweeting

The pins on the Arduino were then set using some of the variables initialized earlier and after that the first main part of the code started. The first main part of the code controlled the basic on and off functionality. This was done by reading the instructions from Processing periodically, due to everything being in an infinite loop, and assigning it to one variable. This variable was then compared with ‘val’ using if statements with different conditions and when the condition was met, the commands in that if statement were performed. A sample of this comparison is shown below in Code 8.

Code 8: comparison of commands

As seen in Code 8, the instructions from Processing were assigned to the variable ‘val’. ‘val’ was then compared with different conditions (the characters correspond to different conditions in Processing) and if a condition was met, the pin associated with that condition is turned ‘HIGH’ or ‘LOW’ and the string that describes what happened is printed out to be read by Processing and tweeted back to the user. Note that a time stamp is added to the string, this is because Twitter does not allow repetitive tweets in a short period of time therefore in case the user performed the same instruction more than once in a short period of time, the time stamp guarantees a different tweet being sent back to the user. The tweet is different in the sense that it provides the same information every time the same information is required but the time stamp provides the difference between tweets needed for Twitter to allow the tweet pass through.

A small feature was also added and that was to check the status of a certain device, Code 9 shows a sample of how this works. Code 9: status check on lights

The second main part of the code was to operate the IR system that detects intruders. Unlike the previous part where the commands were performed upon the user’s request, this system needs to keep on running. To simply do that, the request to perform the IR check was added in the infinite loop without an if statement which means that the check will always be performed as long as the loop keeps on going. The function that checks if the IR signal was cut and alert the user that there is an intruder is shown in Code 10.

Code 10: IR check

The function in Code 10 displays the working of the IR transmission and receiving system. This is discussed in greater detail later in the report. It is implemented in the main loop of the Arduino code and issues an alert back into Processing when the transmission is cut. The string is then read by Processing and tweeted to the user.

The third main part of the code was to operate the temperature sensor. Obtaining the temperature was made upon the user’s request but it can be easily changed to being posted periodically or have both featured. It can be adjusted to personal preference.

Since the temperature is obtained by requesting it, the same method as in the first part of the code was used to execute the function (i.e. comparing the variable ‘val’ with the condition that requests the temperature). The function that will be executed is shown in Code 11 below.

Code 11: Temperature function

The function in Code 11 checks the temperature by reading the value from the temperature sensor and storing it in the variable ‘reading’ then the calculations as shown were performed to convert this value into degrees Celsius. Once the value has been converted, a string is printed that includes the temperature and a time stamp. Processing then reads this string and tweets it to the user.

The full code is attached in Appendix C at the end of the report.

Powering the 12VDC brushless fan

The digital I/O pins of the Uno cards outputs only 5V when high [5]. This voltage is generally very low to power up appliances at home which usually take in 120Vac. The project is established to make the system also compatible with controlling devices that require much less voltage to operate (e.g. 12 V fan) by using a Transistor switcher.

Transistor Switcher

For devices that require low voltage to operate, it would be best to assemble a transistor switcher relay with an external power source. The Arduino digital pins should be able to control the transistor switcher and therefore either connecting the device to its power supply or cut-off the circuit. A transistor switcher is known to operate in the Saturation and the Cut-off Mode. This means that we can overlook the operating Q-point biasing (i.e. Active Region) where BJTs normally operate since the transistor will be switched back and forth between “fully-OFF” and “fully-ON” regions as seen in the figure below.

current flow. The “Vin” node is connected to one of the Arduino’s digital I/O pin,

As seen in figure 4, the base biasing voltage is the factor which decides whether the transistor is off or on. That’s because when current flows from base to emitter, the resistance between collector and emitter drops while there is correspondingly whenever this exact pin is set “high” by the Arduino as a result of a command, the transistor acts as a closed switch and completes the circuit. On the other side, when the Arduino sets that pin low, the transistor will switch to cut-off region and thus opens the circuitry. Vcc is the external power supply that will be used to operate the load when the switch is closed.

It is important to consider all contributing factors when working with transistor switchers. In some application the amount of input current available to switch on a transistor is very low. This may mean that a single transistor may not be able to pass sufficient current required by the load, since the collector current IC is proportional to the base current IB according to the relationship IC =βIB. To overcome this issue it would be best to use a transistor with a very high Beta value. As a result, a transistor switch using multiple transistors is used and this done using the Darlington configuration.

Darlington Transistor Switch

The Darlington Pair is made up of two bipolar transistors connected together in a way so that the current gain (β) of the first transistor is multiplied to that of the second transistor [6]. Therefore, the two transistors behave as one transistor with very high current gain for a smaller Base current. As a result, the Darlington pair was used in the project to control appliances to avoid future failure of switching if the load current/voltage is much higher.  Below is a schematic of the Darlington switcher used by the Arduino to control the 12V fan.

Figure 5 shows the Collector of both NPNs are connected together to the fan and the Emitter Q1 is connected to the base of Q2. When the Arduino sets that pin high, the Emitter current of Q1 becomes the Base current of Q2 and thus both transistors turn on simultaneously. As the current goes through Q2, it is further amplified by βQ2 and results in ΒTotal = βQ1 * βQ2. This causes the transistors current to overcome the load’s current and switch it on.

When the Arduino sets the same pin low, both transistors will be in cut-off region and will act as an open switch. Therefore, despite the external power supply is still on, the fan will turn off since the circuit is incomplete.

D4 is a protection diode used to prevent the fan current from flowing into the emitter of Q2.  Figure 6 below shows the fan set up used in the project, following the schematic shown in figure 5.

Power Switch Tail

The house lamp requires 120V AC to operate, but the Arduino UNO microcontroller can only provide up to 5V DC. A solution to this issue is to use a Power Switch Tail [7], which is an isolated power cord for controlling power to devices with the microcontroller.

Working of the Power Switch Tail

The Power Switch Tail functions like a relay [8], which is an electrically operated switch that consists of an electromagnet. The switch is controlled by an external source and depending on the output from the external source, the switch will either close, complete the circuit and power the device using the high AC voltage source, or vice – versa. Figure 7 below shows the Power Tail’s internal circuitry that performs the function of a relay switch.

Figure 7. Schematic of Internal Circuitry of a Power Switch Tail [9]

In the schematic above in Figure 7, A1 is a phototriac [10] enclosed in a dual in – line package. It is a semiconductor device that uses light to allow small voltages and currents to control (or switch) high voltages. MOV1 and MOV2 are both metal – oxide varistors [11], which are semiconductors that can conduct large currents even when faced with a voltage higher than its rated voltage. These MOV’s protect other electrical devices from high AC voltages by drawing large amounts of currents (produced from the high voltages) to themselves, thus diverting large current surges from other connected devices. K1 is the internal relay switch of the Power Tail. The LOAD SIDE (J1) is where the device or appliance to be tested is connected and the LINE SIDE (J2) is where the high AC voltage source is connected. J3 is the line with which the external controlling source is connected. The three terminals on J3 represent the positive logic input (1), the negative logic input (2) and the electrical ground (3). Lastly, LED1 is a light emitting diode and R1 and R2 are resistors. The LED will turn on and emit light when current flows from its positive terminal to its negative terminal. For information on the LED’s terminals, refer to the section on the light – emitting diode.

When J3 receives a logic “high” signal (which indicates turning the switch ON) from the controlling device, current flows through LED1 (positive to negative) and it emits light. The phototriac, A1, captures the light and permits the current from L3 to flow through it. Once the current passes through A1, it flows into the relay switch K1.

Figure 8. The general design of a relay switch. [12]

The design above helps in explaining the working of a relay switch. Note that the “Control Switch” in the design refers to the external controlling device. When the current enters the relay, it flows through a coil wound around an iron core, which is colored in purple in the design and is represented by two parallel lines and an inductor symbol between terminals 2 and 3 near K1 in Figure 7. Current flowing through the coil induces a strong magnetic field in and around the iron core, thus turning it into a magnet [13]. The line between the power source coming from the Line Side and the Load Side is cut in between by two open metal contacts, which are shown in grey in the design and are represented as terminals 1 and 4 near K1 in Figure 7. The induction of a strong magnetic field around the magnet results in strong attraction between the two contacts, which then come in contact with each other and complete the line between the Line and the Load Sides. This leads to the high voltage source turning on the loaded appliance/device.

The exact opposite result occurs when J3 receives a logic “low” signal from the controlling device. Current flows from the negative to the positive terminal of LED1 instead, thus turning it off and preventing any transmission of light to A1. With no light, A1 prevents current from flowing into K1. This results in the removal of the magnetic field and the return of the metal contacts to their original positions. The connection between the Load and the Line sides is broken and the high voltage source no longer powers the loaded appliance/device.

Figure 9. Design and working of a Metal – Oxide Varistor. [11]

The figures above show the working of a Metal – Oxide Varistor (MOV). The MOV is basically a variable resistor, Rs, with an ideal resistance of infinity under no transient condition, i.e., it is ideally an open circuit in the absence of any transient (or AC) voltages/currents. However, when a transient voltage is provided by the source, a transient current flows through the MOV and it is shorted (ideally), i.e., Rs = 0 ohm’s and it becomes a closed circuit. In reality, this leads to a majority of the current being diverted to Rs, and since Rs and the loaded device are in parallel, the loaded device will have a transient voltage drop across itself. This is how MOV1 and MOV2 function in the PowerSwitch Tail circuit shown in Figure.

One should also note that since the phototriac A1 is driving the relay, the power tail can only be used for AC switching, not DC switching. This is simply because phototriacs only work with AC currents.

Setup

The Power Tail plugs into the standard 120Vac 3-prong outlet (the high voltage source) and the other end is plugged into the lamp (high voltage device). Then a specific digital I/O pin of the Arduino (external source that controls the Power Tail) designated to the lamp will be connected to Pin1 of the Power Tail and a reference or ground from the Arduino is connected to Pin2, as shown in Figure 10 below.

Figure 10. Connections between the Arduino UNO and the Power Switch Tail.

Temperature Sensor Calibration (Luv Tomar)

The Tweet – A – Home System implements a TMP37 [14] temperature sensor to check the temperature of the surrounding environment. According to sensor’s datasheet, it requires a voltage supply range of 2.7V to 5.5V and a supply current of 50 µA. The Arduino UNO microcontroller has a pin that supplies 3.3V and 50 mA. So, there is both enough voltage and enough current to be supplied to the temperature sensor. The sensor itself has 3 pins and the following figure gives an approximate idea of how the pins are arranged.

Figure 11 . Bottom view of a TMP37 Temperature Sensor.

Pin 1 represents the supply voltage connection, pin 2 represents the output connection and pin 3 represents the ground connection. To set up the sensor with the microcontroller, pin 1 is connected to the 3.3V output pin of the microcontroller, pin 2 is connected to one of the analog input pins (A0 to A5) on the microcontroller and pin 3 is connected to the GND (ground) pin of the microcontroller. Figure below shows an example of the TMP37 – Arduino UNO setup.

Figure 12. Sample TMP37 – Arduino UNO setup.

In the sample set up above, pin 1 is connected to the 5V output pin on the microcontroller. However, for the Tweet – A – Home System, the 3.3V pin was chosen instead, as the 5V pin was not giving the correct resulting environment temperature.

The following code (Code 12) was implemented to program the temperature sensor to give the correct temperature reading via Arduino’s IDE: -

Code 12 – Temperature Sensor Calibration – Also shown in Appendix B

The first statement, “int sensorPin = 0”, indicates that the output pin (pin 2) is connected to the A0 pin of the microcontroller. In the comments following the first statement, the resolution is stated to be 20 mV per degree centigrade, as this is the scaling factor given in the sensor’s specifications. The setup () function is used to start the temperature reading and “Serial.begin(9600)” indicates that the serial port that is being used in Arduino’s IDE is numbered 9600. The loop () function is used to constantly repeat the temperature reading as long as the serial connection exists. Within this function, “int reading = analogRead(sensorPin)” is a statement used to read the input value (which can be from 0 to 1023) from the A0 pin on the microcontroller. To convert this voltage reading to a temperature value in degrees Centigrade, the following conversion formulae were used: -

Voltage = (Reading x 3.3)/1024;

Temperature = Voltage/0.02;

Since the minimum and maximum possible values that can be read from the A0 pin are 0 and 1023, there are 1024 possible values for “Reading”. Therefore, to convert it to a voltage value in mV, it is divided by 1024, and it is then multiplied by 3.3 (the supply voltage value). In accordance with the scaling factor of the TMP37 sensor, “Voltage” is divided by 20 mV to give the resulting “Temperature” value in degrees Centrigrade. The following commands are then used to print the temperature reading in a proper format in the serial monitor: -

Serial.print(temp);

Serial.println(" degrees C");

Finally, at the end of the repeating loop () function, a delay of 1 second is added for a steady, yet constant display of the temperature reading in the serial monitor: -

delay(1000);

Note that the time unit in the command above is ms (i.e., 1000 ms = 1 s).

IR Intrusion Alert System:

One of the features implemented in the “Tweet-A-Home” project was the IR Intrusion Alert System. This is a low level security system implemented using an infrared connection to demonstrate the capabilities of the “Tweet-A-Home” in terms of home security.

Background:

IR (Infrared) is a form of wireless communication technology. IR light has a longer wavelength than the visible light spectrum so it is invisible to the naked eye, which makes it useful to implement in low level security systems [15]. However the light can be viewed when seen through the lens of most cell phone cameras. There are many uses involving IR, but it is commonly used in almost all remotes to control house hold appliances such televisions, stereo systems, portable heaters and etc... The receivers in most of these appliances receives signals modulated at 38 KHz [16], since hardly any other sources emit at this frequency, it is mostly reserved for these types of appliance remotes.

Figure13 . IR (LED) Photo Diode and IR Photo Transistor Pair

Design and Implementation:

The IR emitter/sensor set needed to be set up in specific way for it to function. Once the set up was completed, a signal would transmit continuously. Simultaneously the same signal would be received with the IR photo transistor sensor. If the signal between them was cut then an alert tweet would be sent to the twitter user saying “Intruder at Home”. This system is basically trying to imitate an alarm system where as if an intruder did come in, then an alarm would be tripped.

Hardware Setup:

For the configuration of this system a 5 mm IR LED photo diode emitter and a matched IR photo-transistor sensor was used for establishing the IR signal. The components required for this configuration was really minimal. Just the IR emitter/sensor pair and two resistors were required with a value of 150 ohms and 8.2 Mega Ohms respectively.

Figure 14. Hardware Setup for the IR Intrusion System

The configuration was done exactly as shown above (refer to Figure 14) on a bread board. The Vss signal came from Arduino pin 8. That pin basically turned high or low depending on the software implemented. When high, a +5V power signal was supplied through the Arduino output pin 8 to the IR photo diode and the IR photo-transistor. The current supplied to the IR photo diode was limited by the 150 ohm resistor so as not to burn out the LED. The IR photo diode had a forward voltage drop of 1.2V so enough power was supplied to the LED to transmit enough light to be received. The phototransistor contained an exposed silicon junction that allowed most non-IR light to pass through. But only lights with IR wavelengths were absorbed by the sensor within [17]. Every time IR light was received by the photo transistor, current started to flow, this tiny current was then amplified by the transistor’s current gain. The current was then forced to go through the 8.2 Mega Ohm resistor. The current passing through the resistor caused the voltage value across the resistor to fluctuate with this V=IR relationship. The change in voltage across the resistor was then read by the Arduino Microcontroller input analog Pin A0. Both grounds connected to the Arduino universal ground pin.

In this setup as long the Arduino was outputting a high +5V power signal to the IR photo diode and IR photo transistor then an IR connection would be established between those two components. But if for some reason the IR signal across the components was cut then the Arduino would have read some other value inferring an intrusion alert.

Software Setup:

Code was written for the Arduino microcontroller so that it could control the IR Security Intrusion system.

Code 13. Software setup for the IR Intrusion System

The code written for the Arduino Uno microcontroller to control the IR Intrusion system is shown above (Figure 3).  Two pins from the Arduino microcontroller were designated for this part. Pin 8 was designated to be the output pin that would send a constant high signal of +5V to power the IR photo diode and IR photo transistor and establish the IR connection between them. Analog pin A0 was designated to be the input pin that read the voltage change across the 8.2 Mega Ohm resistor when the IR light was received by the IR photo transistor. In the IR function written in the code, the read value was stored in a variable called value, this variable was compared to values between 200 – 500, and if the value fell within that range then a tweet would be printed on the console which would be then sent to the twitter user warning of an intrusion.  The reason why a range of below 500 was chosen for the alert to be sent was because we observed that when the IR signal was being transmitted and received, the value read was really high in the 2000 range but once the signal was cut the value dropped down to 200-500 range. The reason why the value did not drop to zero was because the IR photo transistor picked up external IR light.

Notes:

There were few things to keep in mind when implementing this system. Depending on where the configuration was set, the compared value ranges in the code might have to have been changed because different environments would have different IR noises. The two resistors used in the hardware configuration could also be changed. A low resistance could have been used to limit the current to the IR photo diode if a higher IR light intensity was required to be transmitted. A lower value resistor could have also replaced the resistor between the IR photo transistor and ground. It all depends how sensitive of a read is required from the voltage change.

Specifications and Project Cost:

Here is a list of the specifications for the Tweet-A-Home system:

- Multi Mobile Device Capability

- Arduino Uno R3

- Chipset: Atmega 328

- Operating Voltage: 5V

- Input Voltage Limits: 6-20 V

- Temperature Sensor range :

- -40°C to 125°C

- Accuracy: ±1 oC (max ±2 °C)

- TMP 37: 2.7 to 5.5 Vdc

- Operating Voltage of Devices:

- Desk Lamp : 120Vac (150 mA)

- DC Fan: 5V (190 mA)

- Red LED: 1.8 to 2.2 Vdc

- Infra-red Sensor/Detector:

- Forward Voltage: 1.2V (max 1.6V)

- Wavelength: 940 nm

- Forward Current: 50mA

- Input Voltage for the Relay

- DC input: 3-12V (3-30mA)

- AC Input/Output: 120V (≤15 A)

- Tweet (Commands) Character Limit: 140

- Number of commands sent simultaneously: 1

Project Cost

These are the main components required to construct this system:

| - Arduino Uno | - Temperature Sensor | - DC brushless fan |
| --- | --- | --- |
| - Jumper Wires | - Infrared Sensor | - LED |
| - DC activated relay | - Desk Lamp |  |

The total cost of the inventory required : $31.50
The Arduino Uno, Desk Lamp and Infrared Sensor are items were already in the group’s inventory, but by including these items, the total cost would be: $68.50

Conclusion and Future Work:

The “Tweet-A-Home” project is not just another home automation product. It is revolutionary in its own way because it allows the user endless customizability features. In an age where social networking is big part of almost every individual life with users using sites such as Twitter or Facebook to constantly update their statues, post photos, and interact with other individuals. But these sites should not be only limited to that, the “Tweet-A-Home” project allows users to use social networking site such as Twitter to do much more, to control real life appliances and device, to get status updates on electronic devices in your and tweet to them to make them function. With our “”Tweet-A-Project” we believe we accomplished that goal. The “Tweet-A-Home” project functioned perfectly; all the appliance demonstrations were successful.

In our project basic appliances were used for the demonstration to mimic actual appliances that would be used in a regular household. The temperature sensor represented the thermostat of a house, the DC brushless fan mimicked the cooling system in the house, the lamp represented the lighting system, and finally the IR intrusion system represented the security system that could be implemented in a home. For future work, this “Tweet-A-Home” project could be directly implemented in actual home controlling actual appliances. For this, the Arduino microcontroller would have to be replaced with a more powerful, versatile, with a lot more memory microcontroller. Also instead of connecting the micro controller to the home computer or network, a wireless feature could be added to the microcontroller so that it could wireless connect to the home network from where ever it is placed. In terms of the appliances it could be made to control, there are a lot possibilities ranging from controlling your television to controlling you coffee machine through Twitter.

Appendix B – Processing Code

import twitter4j.conf.*;

import twitter4j.*;

import twitter4j.auth.*;

import twitter4j.api.*;

import java.util.*;

import processing.serial.*;

List<Status>statuses = null;

TwitterFactory twitterFactory;

Twitter twitter;

Serial myPort;

String LastTweet="";

String ArduinoMsg="x"; // Response from arduino

String ArduinoTemp="x";

void setup() {

size(800, 600);

background(0);

connectTwitter();

String arduinoPort = Serial.list()[1];

myPort = new Serial(this, arduinoPort, 9600);

}

//displays the last posted tweet in a window

void draw() {

background(0);

getTweets();

responseCheck();

delay(5000);  // Checks twitter every 10 seconds, to avoid exceeding the frame limit , the delay is not made shorter.

}

// Initial connection

void connectTwitter() {

ConfigurationBuilder cb = new ConfigurationBuilder();

cb.setOAuthConsumerKey("npdCfxKDBfS8wNd473pJw");  //Consumer Key

cb.setOAuthConsumerSecret("TP6MhaSP82nc17TfR8wTUlFm7LOJd5Pq1CoklAnQc0"); //Consumer Secret

cb.setOAuthAccessToken("2307092941-qpZBBH8camPy33hPYGVDbbG5b2Wy8DIZCzAhU0F");  // Access Token

cb.setOAuthAccessTokenSecret("A9dqQxbkiL3fPk7qE4LETacI78XRKzyp2Hbi6mV4vg1Ja");  // Access Token Secret

twitterFactory = new TwitterFactory(cb.build());

twitter = twitterFactory.getInstance();

println("connected");

}

// Method to get tweets

void getTweets() {

try {

statuses = twitter.getUserTimeline();

}

catch(TwitterException error) {

println("Get timeline: " + error + " Status code: " + error.getStatusCode());

}

text(statuses.get(0).getUser().getName() + ": " + statuses.get(0).getText(), random(width), random(height), 300, 200);

LastTweet= statuses.get(0).getText();

if (LastTweet.equals("Turn on the lights"))

{

myPort.write('1');         //sends a 1 as a character to compare, as a result turn lights on.

println("lights is on");

}

else if (LastTweet.equals("Turn off the lights"))

{

myPort.write('2');          //sends a 2 as a character to compare, as a result turn lights off.

println("lights is off");

}

else if( LastTweet.equals("Turn on the fan"))

{

myPort.write('3'); //sends a 3 as a character to compare, as a result turn the fan on high speed.

println("fan is on");

}

else if( LastTweet.equals("Fan on medium"))

{

myPort.write('4'); //sends a 4 as a character to compare, as a result turn the fan on medium speed.

println("fan is on");

}

else if( LastTweet.equals("Turn off the fan"))

{

myPort.write('5'); //sends a 5 as a character to compare, as a result turn the fan off.

println("fan is off");

}

else if( LastTweet.equals("Turn on the heater"))

{

myPort.write('6'); //sends a 6 as a character to compare, as a result turn the heater on.

println("heater is on");

}

else if( LastTweet.equals("Turn off the heater"))

{

myPort.write('7'); //sends a 7 as a character to compare, as a result turn the heater off.

println("heater is off");

}

else if( LastTweet.equals("Status of the lights"))

{

myPort.write('8'); //sends a 8 as a character to compare, as a result checks lights.

println("light check");

}

else if( LastTweet.equals("Status of the fan"))

{

myPort.write('9'); //sends a 9 as a character to compare, as a result checks fan.

println("fan check");

}

else if( LastTweet.equals("Status of the heater"))

{

myPort.write('0'); //sends a 0 as a character to compare, as a result checks heater.

println("heater check");

}

else if( LastTweet.equals("What is the current temperature?"))

{

myPort.write('t'); //sends a 't' as a character to compare, as a result check temperature.

println("temp check");

}

delay(1000);

myPort.write('x');

}

// Method to post tweets

void postTweet(String S)

{

try

{

Status status = twitter.updateStatus(S);

println("new tweet --:{ " + status.getText() + " }:--");

}

catch(TwitterException e) {

println("Status Error: " + e + "; statusCode: " + e.getStatusCode());

}

}

// Checks for response from Arduino and posts the relevant tweet

void responseCheck()

{

delay(200);

ArduinoMsg=myPort.readStringUntil('\n');

if(ArduinoMsg!=null){

if(!ArduinoMsg.equals(ArduinoTemp)){

postTweet(ArduinoMsg);

}

ArduinoTemp=ArduinoMsg;

}   
}

Appendix C – Arduino Code

// The following variables are used to assign the input and output pins

char val; // Data received from the serial port

int lights = 13;

int fan = 11;

int heater = 7;

int IRpin=8;

int IRsensor=A0;

int value=1;

int state=0;

int tempPin = 1; // Temperature sensor pin

int reading = 0;

float volts = 0;

float temp = 0;

unsigned long time;

String Str1="Current Temperature of the room: "; // Temporary string needed for temperature readings.

String Str2=""; // Temporary string needed for temperature readings.

String Str3=" degree celsius";

// The following strings are used to tweet back to the user.

String LightStr1="Lights are on";

String LightStr0="Lights are off";

String FanStr1="Fan on high mode";

String FanStrMed="Fan on medium mode";

String FanStr0="Fan is off";

String Heater1="Heater is on";

String Heater0="Heater is off";

String LightCheckStr1="Lights are still on";

String LightCheckStr0="Lights are still off";

String FanCheckStr1="Fan is still on";

String FanCheckStr0="Fan is still off";

String IRStr="Intruder at home.";

String Hashtag=" #";

String TempStr="";

void setup() {

pinMode(lights, OUTPUT); // Set pin as OUTPUT

pinMode(fan,OUTPUT);

pinMode(heater, OUTPUT);

pinMode(IRpin,OUTPUT);

pinMode(IRsensor,INPUT); // Set pin as INPUT

Serial.begin(9600); // Start serial communication at 9600 bps

}

void loop() {

time=millis();

time=time/1000;

String TimeStr= String(time);

if (Serial.available())

{ // If data is available to read,

val = Serial.read(); // read it and store it in val

}

if (val == '1')

{ // If 1 was received

digitalWrite(lights, HIGH); // turn the lights on

TempStr=LightStr1+Hashtag+TimeStr; // adding a time stamp to the string

Serial.println(TempStr); // printing the string where it will be read by Processing

}

else if(val == '2')

{

digitalWrite(lights, LOW); // turn lights off

TempStr=LightStr0+Hashtag+TimeStr;

Serial.println(TempStr);

}

else if (val == '3')

{

digitalWrite(fan, HIGH);

TempStr=FanStr1+Hashtag+TimeStr; // turn fan on at high mode

Serial.println(TempStr);

}

else if (val == '4')

{

analogWrite(fan, 130); // turn fan on at medium mode, PWM is used here to slow down the fan

TempStr=FanStrMed+Hashtag+TimeStr;

Serial.println(TempStr);

}

else if (val == '5')

{

digitalWrite(fan,LOW); // turn off the fan

TempStr=FanStr0+Hashtag+TimeStr;

Serial.println(TempStr);

}

else if (val == '6')

{

digitalWrite(heater, HIGH); // turn on the heater

TempStr=Heater1+Hashtag+TimeStr;

Serial.println(TempStr);

}

else if (val == '7')

{

digitalWrite(heater,LOW); // turn off the heater

TempStr=Heater0+Hashtag+TimeStr;

Serial.println(TempStr);

}

else if (val == '8') // Status check for lights

{

state=digitalRead(lights); // the state of the light is assigned to the variable 'state'

if (state==1) // if the state is high

{ TempStr=LightCheckStr1+Hashtag+TimeStr;

Serial.println(TempStr); // string saying the lights are on in printed for Processing to read

}

else // if the state is low

{ TempStr=LightCheckStr0+Hashtag+TimeStr;

Serial.println(TempStr); // string saying the lights are off in printed for Processing to read

}

}

else if (val == '9') // Status check for fan

{

state=digitalRead(fan);

if (state==0)

{ TempStr=FanCheckStr1+Hashtag+TimeStr;

Serial.println(TempStr);

}

else

{ TempStr=FanCheckStr0+Hashtag+TimeStr;

Serial.println(TempStr);

}

}

else if (val == '0') // Status check for heater

{

state=digitalRead(heater);

if (state==0)

{ TempStr=Heater1+Hashtag+TimeStr;

Serial.println(TempStr);

}

else

{ TempStr=Heater0+Hashtag+TimeStr;

Serial.println(TempStr);

}

}

else if (val == 't')

{

Temp(); // checks the temperature

}

IR(); // always checks if the IR signal was cut

delay(1000); // Wait 10 milliseconds for next reading to avoid overloading Twitter servers

}

void IR()

{

time=millis();

time=time/1000;

String TimeStr= String(time);

digitalWrite(IRpin, HIGH); // IR transmitter is always on

value= analogRead(IRsensor); // the value read by the IR receiver is assigned to the variable 'value'

delay(100);

// when there are no obstacles between the IR transmitter and receiver

//the value read by the receiver fluctuates around 900

// when the signal is cut, the value read by the receiver fluctuates between 300 and 400

if(value>200 && value <500) // i.e. if the signal is cut

{

TempStr=IRStr+Hashtag+TimeStr;

Serial.println(TempStr); // print saying that there is an intruder.

}

}

void Temp()

{

reading = analogRead(tempPin); // reads and stores the value from the temperature sensor to the variable 'reading'

// converting that reading to voltage, for 3.3v arduino use 3.3

volts = (reading * 3.3 / 1024.0);

temp = volts/0.020;

//after the conversion, the following string is printed along with the temperature in degrees celsius and a time stamp

Serial.print("Current temperature of the room:");

Serial.print(temp);

Serial.print(" degrees C #");

Serial.println(String(millis()/1000));

// the value obtained from reading is between 0 - 1023, corresponding

// to the analog input voltage at the AIO pin between 0V - 3.3V

}

References

[1] Atmel (2014, March 12). ATmega328P [online]. Available: http://www.atmel.ca/devices/ATMEGA328P.aspx

[2] Canada Mortgage and Housing Corporation (2014, March 13). Home Automation [online]. Available: http://www.cmhc-schl.gc.ca/en/co/acho/acho_011.cfm

[3] Twitter (2014, March 13). Twitter Developers [online]. Available:  https://dev.twitter.com/

[4] Twitter4J (2014, March 13). Twitter4J JavaDoc [online]. Available: http://twitter4j.org/en/javadoc.html

[5]  Arduino (2014, March 12). Arduino Uno [online]. Available: http://arduino.cc/en/Main/arduinoBoardUno#.Uxno6ZKsiSo,

[6] Jay Summet (2014, March 12). Darlington Pair [online]. Available: http://www.cc.gatech.edu/classes/AY2011/cs3651_spring/docs/Darlington_Pair.pdf

[7] PowerSwitch Tail (2014, March 16). PowerSwitch Tail II [online]. Available:  http://www.powerswitchtail.com/Pages/default.aspx

[8] Wikipedia (2014, March 16). Relay Switch [online]. Available: http://en.wikipedia.org/wiki/Relay

[9] mBed (2014, March 20). PowerSwitch Tail II [online]. Available: http://mbed.org/users/4180_1/notebook/powerswitch-tail-ii/

[10] Vishay (2014, March 20). Phototriac [online]. Available: http://www.vishay.com/docs/84780/appnote34.pdf

[11] ElectroSchematics (2014, March 20). Metal Oxide Varistor [online]. Available: http://www.electroschematics.com/5224/metal-oxide-varistor/

[12] Circuits Today (2014, March 20). Working of Relays [online]. Available: http://www.circuitstoday.com/working-of-relays

[13] Wikipedia (2014, March 20). Electromagnetic Induction [online]. Available: http://en.wikipedia.org/wiki/Electromagnetic_induction

[14] Digikey (2014, March 12). TMP37 Sensor Datasheet [online]. Available:   http://www.digikey.com/catalog/en/partgroup/tmp35-tmp36-tmp37/10726

[15] Wikipedia. (2014, February 4). Infrared (IR) [online]. Available:  http://en.wikipedia.org/wiki/Infrared

[16] Spark Fun. (2014, February 28).  IR Communications [online] Available: https://learn.sparkfun.com/tutorials/ir-communication/all

[17] Nerd Kits. (2014, March 5). Theremin with IR Distance Sensor [online] Available: http://www.nerdkits.com/videos/theremin_with_ir_distance_sensor/